### Handouts:

- Iron ECU Pinout

### Course 1 (40 hours, 5 hrs/day x 8 days)

GOALS: after day 8 the student should be able to:

- identify EFI components

- build a minimal bench harness

- connect laptop → ECU → read values

- make a simple AFR/spark change and understand the result

## COURSE 1 (revised)

### DAY 1 — EFI SYSTEM OVERVIEW + COMPONENT IDENTIFICATION

classroom

- Que es un osciloscopio? Voltage vs tiempo, tiempo es importante porque podemos ver fallas de sensores si el voltaje no se comporta como esperamos

- what EFI is vs carburetor - talk about fuel delivery and the different way different (1hr)technologies handle it.

  - Start with “que es EFI? Porque se decidio cambiar de carburadores a EFI? Para hablar de EFI tenemos que entender carburadores y sus limitaciones”

    - Carburadores (15min)

    - Inyeccion mecanica y sus fallas (5min)

    - Questions? (5min minimum)

  - EFI y sus ventajas

    - TBI (throttle body injection, primeros sistemas, drawbacks y beneficios como su diseno facil de implementar pero la inyeccion es pobre; porque le llaman inyector de ducha) (20min)

    - MPFI (multi point fuel injection, ventajas de multiples inyectores, cambios de presion, precision por cilindro, etc) (ITB should be here) (20 min)

    - Direct injection (nuevas ventajas y drawbacks, indicar que la mayoria de los sistemas hoy dia son mpfi) (15min)

    - Questions?

  - Hablar un poco de la mejoras y la configurabilidad que te da EFI vs carburador (10min)

- what components exist: ECU, injectors, coil(s), MAP, TPS, CLT, IAT, WB, crank sensor

  - Sensor per sensor, 5 minutes per sensor, explain basically how it works

    - Clt/iat = thermic resistor, increasing temperature decreases resistance, show IRL

    - TPS = variable resistor, increasing TPS increases resistance to ground, increasing bias to 5v, show IRL

    - Crank sensor = hall sensors switch to ground when metal is detected, VR produce an induced sine wave

    - Map sensor = sensing element reduces resistance to 5v, increasing voltage

    - MAF = flowmeter = different MAF produce different waves

    - Questions?

- what each sensor does in the ECU math

  - clt/iat correction

  - Map sensor = main load sensor that tells us the pressure we use in PV = nRT

  - Talk about MAF and AlphaN

  - Narrowband o2 = more fuel causes sensor to produce close to 1v. Less than stoich goes to 0v

  - Each part should be about 10min

  - Questions?

Quiz

30 minutes

lab

- physically handle each sensor

- test TPS with multimeter (0–5V sweep)

- test MAP sensor vacuum = voltage drop

- Test crank sensor -> put close to metal object and observe waveform

### DAY 2 — POWER, GROUNDS & BASIC WIRING PRACTICE

classroom

- 12V automotive power — where it comes from

- Why 5v for sensors

- Digital vs analog sensors

- main relay, fuel pump relay; why no polarity

- grounds → why it matters more than the power wire

lab

- build a simple relay circuit

- measure voltage drop with 2 meters: ECU GND vs chassis GND

- Make a small harness with IAT/CLT/MAP/TPS/CKP

### DAY 3 — BENCH ECU POWER-UP + TunerStudio / AlphaECU BASIC CONNECT (using AlphaECU Iron, Laptop)

classroom

- USB vs serial vs CAN

- Laptop recommendation windows 7+, thinkpad prefereably

- firmware vs tune file

- how to read “live data”

lab

- power an ECU on the bench with 12V supply

- connect laptop → ECU

- see live TPS, CLT, MAP values changing (manually blowing on IAT, finger-press TPS, vacuum pump MAP)

### DAY 4 — INJECTORS, IGNITION, WHAT DWELL/PW ACTUALLY IS (need iron harness)

classroom

- what injector pulse width means

- what dwell means

- how spark is generated

lab

- fire a single injector with test pulse generator (or ECU test mode)

- measure injector click & voltage waveform

- run a bench ignition coil with dwell change and see effect on current ramp on scope

### DAY 5 — FUEL TABLE & IGNITION TABLE BASICS

Classroom (guide using Cell table to demonstrate)

- VE table orientation (rpm vs load)

- timing table structure

- How is timing, calibration timing light

- rich vs lean vs knock safe

lab

- open a .msq file (or alphaecu tune) Use any file provided by Norman)

- deliberately change 2 cells

- compare log before/after; look at changes in oscilloscope

### DAY 6 — STARTUP CAL, AFR TARGETS, BASIC TUNING LOGIC

classroom

- cold start enrichment

- warm-up enrichment

- AFR targets table - explain and demostrate VE, speed density

- how O2 feedback works

- Accel enrich

lab

- edit AFR target table

- replay sample log files → adjust VE to match targets

- Use tune analyzer to simulate real behavior

### DAY 7 — BASIC TROUBLESHOOTING: “WHY DOESN’T IT START?”

classroom

- no spark faults (learn how identify by senses)

- no fuel faults

- trigger issues(

- ground reference faults

lab

- fault board / “pre-broken harness” (Alpha x will provide BUG harness)

  - 1 bad ground

  - 1 reversed TPS pins

  - 1 injector not powered

- students must diagnose w/ multimeter + TS live data

### DAY 8 — FINAL PRACTICAL / MINIMUM FUNCTIONAL BENCH ECU

classroom

- review concepts

- when to say “this is tuning” vs “this is wiring”

lab (capstone)

- student must:

  - build a small harness subset (TPS + MAP + power)

  - power up ECU

  - connect laptop

  - show working live values

  - change fuel table 2% and save tune

pass = you can actually run a laptop + ECU + harness on bench

Required materials for 24 students (12 items each):

- AlphaECU Iron

- Laptop with Windows 7+

- 18AWG 30CM Molex 5557 Series 4.2 mm 2x6pin5557-22R Micro-Fit 4.2 Housing 10P  (12 pin Molex 4.2 2*6pin 12p 24P wire harness - AliExpress <- need 40, 12 for good harness, 12 for bad harness, 12 for lab projects where student does their own wire.

- 12585546 Crankshaft Position Sensor For Chevy GMC Cadillac - Avalanche Camaro Silverado Suburban Tahoe Envoy Savana Sierra Yukon - AliExpress <- crank sensor

- FNIRSI New DSO510 DSO152 DSO153 Mini Handheld Digital 2 IN 1 Oscilloscope Portable 48MS/s 10MHz Bandwidth Electronic Repair Tool - AliExpress 1420 <- oscope

- High Quality 100% Test Manifold Intake Air Charge Temperature Temp Sensor Sender For IAT MAT ACT 25037225 25036751 25037334 - AliExpress <- iat

- 89422-87102 Coolant Water Temperature Sensor & Wire Fits For Daihatsu Terios Toyota 8942287102 89422 87102 - AliExpress <- clt

- 13420-65D01 Throttle Position Sensor TPS Plug For Suzuki Grand Vitara Ignis Jimny 1342065D01 13420-52D00 1342052D00 - AliExpress <- tps

- https://www.aliexpress.com/item/32848965670.html <- 2bar map

- JD2912 4 Pin 5 Pin 40A Auto Car Relay DC 12V 24V 36V 48V with Mounting Hole Coil Power 1.8W Automobile Control Device with Line - AliExpress <- relay

- https://www.aliexpress.com/item/1005009846443506.html <- fuel injector (need pigtail!)

- https://www.aliexpress.com/item/1005008634619269.html <- ignition coil

- 6A 120W Super Fast Charging USB Type C Data & Sync Cable For Samsung S23 Realme Xiaomi 13 Huawei Mobile Phone Quick Braid Cord - AliExpress <- iusbc cables

- https://www.aliexpress.com/item/1005007264933490.html? Power suply