# DAY 2 — POWER, GROUNDS & BASIC WIRING PRACTICE (5 HOURS TOTAL)

## CLASSROOM — 2 hours 30 minutes (150 min)

### 1. 12V Automotive Power — Where It Comes From (30 min)

- – The battery supplies stable low-noise power when the engine is off and during cranking.

- – The alternator provides power once running and keeps the battery charged.

- – Voltage in a “12V system” actually varies between ~11–14.5V.

- – Ignition switch positions (ACC/ON/START) determine what circuits receive power.

- – Fuses and fusible links protect the wiring from shorts.

### 2. Main Relay & Fuel Pump Relay (30 min)

- – Relays let the ECU control high-current loads using a low-current signal.

- – Main relays power the ECU, injectors, coils, and 5V regulators.

- – Fuel pump relays are activated by the ECU only when needed for safety.

- – Understanding pin numbers (85/86 coil, 30/87 switch) makes wiring easier.

- – Common failure modes include burnt contacts, weak coils, or stuck relays.

### 3. Grounds — Why They Matter More Than the Power Wire (40 min)

- – All electronics depend on a solid ground reference; voltage is meaningless without it.

- – Bad grounds create voltage drop, noise, and random sensor errors.

- – Sensor grounds must stay clean and separate from high-current grounds.

- – Poor grounds can cause intermittent stalling, misfires, and false readings.

- – Star-grounding helps avoid loops and stabilizes the ECU’s reference point.

### 4. Classroom Q&A (10 min)

- – Address any confusion before moving on to hands-on tasks.

### 5. Safety, Tools & Wiring Best Practices (40 min)

- – Wire gauge selection is critical for current handling and voltage drop.

- – Crimping is preferred in automotive for vibration resistance.

- – Heat-shrink and loom protect wires from abrasion, heat, and fluids.

- – Learn to identify common connector families (Bosch EV1, Delphi, Sumitomo).

- – Reverse polarity or shorts can instantly damage sensors or ECUs.

## LAB — 2 hours 30 minutes (150 min)

### Phase 1: The Build (25 Minutes)

Goal: Master the SN-2549 and "Open Barrel" terminals.

- The Harness: Students must crimp four wires for a standard 4-pin relay:

  - Pin 30/87 (High Current): Caliber 12/14 AWG.

  - Pin 85/86 (Trigger): Caliber 20/22 AWG.

- The Technique: Focus on the "Double Crimp." The instructor must verify that the "wings" are gripping the insulation for strain relief.

- The Load: Connect Pin 87 to a high-draw DC motor or a large halogen bulb (something that creates electrical "noise").

### Phase 2: The Oscilloscope Anatomy (15 Minutes)

Goal: See the "Voltage Spikes" and "Flyback" mentioned in Section 2.5.

- Back-EMF Observation: Probe Pin 85 (the trigger) while clicking the relay on and off.

  - What to look for: The massive voltage spike (Inductive Kick) that happens the moment the coil de-energizes.

- The Lesson: This shows why we use Flyback Diodes. If this spike travels back to the ECU, it can "reset" the processor or damage a driver.

### Phase 3: The "Ghost in the Machine" Experiment (20 Minutes)

Goal: Prove why Section 3.1 (The Muro de Berlín) exists using a "Live Error" simulation.

- Setup: Connect a simple 5V TPS (Throttle Position Sensor) or a Potentiometer to a clean power supply. Set the Scope to Ch.1 on the Sensor Signal.

- The "Bad" Ground: Intentionally ground the Sensor Ground to the same bolt/wire as the High-Current Load (the motor from Phase 1).

- The Discovery: Turn the motor/load on.

  - The Result: Students will see the "clean" 5V signal on the scope start to jump, vibrate, or offset (The Ground Offset from Section 3.2).

  - Math Check: If the signal was 0.5V and it’s now jumping to 0.7V, ask the students: "If this were your Oil Pressure sensor, would the ECU think the engine is safe or dying?"