# DAY 3 — BENCH ECU POWER-UP + TunerStudio / AlphaECU BASIC CONNECT

### (Using AlphaECU Iron + Laptop)

TOTAL: 5 hours (300 minutes)

# CLASSROOM — 2 hours 30 minutes (150 min)

### 1. USB vs Serial vs CAN (40 min)

- – USB: Modern, plug-and-play, used for flashing firmware and connecting to TunerStudio.

- – Serial (RS232): Old but very reliable; many early ECUs still require serial adapters.

  - Dive a bit deeper into serial, different speed (baud), and how do serial-USB converters affect communications vs talking raw serial.

- – CAN (Controller Area Network): High-speed messaging used by OEM ECUs; AlphaECU supports it. Usually used for inter-control unit communications

- **– Differences in speed, noise immunity, and cable length limitations.

- – Why some ECUs offer all three, and when each is used.

### 2. Laptop Recommendations (20 min)

- – Windows 7 or newer is ideal due to driver compatibility with TunerStudio and USB chipsets.

- – ThinkPads recommended because of strong USB ports, reliable power delivery, and rugged design.

### 3. Firmware vs Tune File (40 min)

- – Firmware: The operating system inside the ECU; defines features, inputs, logic, and outputs. Give examples of a firmware

- Explain why “map corrption” is almost impossible, why “losing maps” when disconnecting the battery is almost impossible (ECU’s use ROM), explain RAM and what it’s used for in ECU’s.

- – Tune file: The calibration data — fuel table, spark table, MAP curve, sensor types, corrections, etc. Give example of a tune file

- **– Updating firmware may require reloading or migrating the tune file.

- – Explain why mismatching firmware/tune versions can cause issues.

### 4. How to Read Live Data in TunerStudio / AlphaECU (40 min)

- – Live data shows real-time sensor values from the ECU.

- – Identifying normal vs abnormal TPS/CLT/IAT/MAP readings.

- – Understanding refresh rate, data lag, and noise.

- – How to change dashboards, gauges, and data views.

- – Importance of verifying all sensor values before attempting to start an engine.

### 5. Classroom Q&A (10 min)

- – Allow students to clarify software questions before lab.

# LAB — 2 hours 30 minutes (150 min)

### 1. Power an ECU on the Bench Using a 12V Supply (30 min)

- – Identify proper +12V and GND pins on the AlphaECU Iron bench harness.

- – Students learn safe power-up procedure (power → wait → confirm LED behavior).

- – Check for errors: reverse polarity, weak power supply, poor ground.

- Use oscope to corroborate

### 2. Connect Laptop → ECU (USB or Serial) (30 min)

- – Install drivers if needed.

- – Open TunerStudio / AlphaECU and detect the ECU.

- – Set the correct COM port or USB connection.

- – Verify communication by observing the “online” indicator.

### 3. Observe Live Sensor Values (TPS / CLT / MAP / IAT) (60 min)

- – Move TPS by hand and confirm voltage sweep (0% → 100%).

- – Warm CLT/IAT by hand or breath and watch temperature change.

- – Use a vacuum pump to see MAP sensor drop from 100 kPa downward.

- – Teach students what “normal” values look like for each sensor.

- – Troubleshoot common issues: flipped wires, wrong sensor type, 5V faults.

### 4. Lab Review + Cleanup (30 min)

- – Confirm every ECU connects reliably.

- – Review sensor behavior and discuss any abnormal readings.

- – Shut down ECUs safely and store laptops, cables, and power supplies.