# DAY 4 — INJECTORS, IGNITION & WHAT DWELL/PW ACTUALLY IS

### (Requires AlphaECU Iron + bench harness)

TOTAL: 5 hours (300 minutes)

# CLASSROOM — 2 hours 30 minutes (150 min)

### 1. What Injector Pulse Width (PW) Means (45 min)

- – Pulse width = the amount of time the injector is held open.

- – More PW = more fuel; less PW = less fuel.

- – ECU calculates PW using MAP/MAF, RPM, VE, and corrections (IAT/CLT/etc.). Compare Speed Density/ MAF/ AlphaN

- – Typical idle PW is ~1.0–4 ms; WOT can reach 12–18 ms depending on setup.

- – Explaining injector deadtime and why actual fuel flow isn’t linear.

- – Show how battery voltage affects injector opening speed.

### 2. What Dwell Means (45 min)

- – Dwell = the time the ECU charges the ignition coil before firing it.

- – More dwell = stronger spark, but too much overheats coils.

- – Dwell depends on coil type (smart/logic, dumb, inductive) and battery voltage.

- – Explain inductive ramp: current rises, saturates, then becomes waste heat.

- – Too little dwell = weak spark; too much = damaged coil or driver.

### 3. How Spark Is Generated (50 min)

- – Current is stored in the coil's primary winding → magnetic field builds.

- – When ECU cuts current, field collapses → high-voltage spike in the secondary.

- – HV from secondary jumps the plug gap → spark.

- – Coil design: inductance, saturation current, turns ratio.

- – Overview of ignition modes: distributor, wasted spark, coil-on-plug.

- – Role of ignition drivers (IGBT / smart coil internal driver).

### 4. Classroom Q&A (10 min)

- – Clarify injector/ignition concepts before moving to bench testing.

# LAB — 2 hours 30 minutes (150 min)

### 1. Fire a Single Injector Using Test Pulse Generator or ECU Test Mode (45 min)

- – Students connect injector to AlphaECU Iron harness.

- – Trigger different pulse widths (1 ms, 3 ms, 6 ms, etc.).

- – Observe spray sound differences (click strength, frequency).

- – Discuss why small PW differences are critical at idle.

### 2. Measure Injector Click & Voltage Waveform on Scope (45 min)

- – Use oscilloscope to view injector voltage and flyback spike.

- – Identify opening notch and closing spike.

- – Compare behavior with low vs high PW.

- – Show how deadtime appears on the waveform.

### 3. Run a Bench Ignition Coil & Change Dwell (45 min)

- – Use ECU test mode to run a coil at various dwell values (1 ms → 6 ms).

- – Observe primary current ramp on the oscilloscope.

- – Identify saturation point when the coil stops gaining spark energy.

- – Discuss coil heating and why too much dwell kills coils.

- – Compare logic-level coil vs dumb coil behavior if available.

### 4. Lab Review + Cleanup (15 min)

- – Confirm students can read injector and coil waveforms.

- – Summarize PW and dwell safety limits.

- – Pack up ECUs, injectors, coils, scopes, and harnesses.