# DAY 5 — FUEL TABLE & IGNITION TABLE BASICS

### (Using TunerStudio / AlphaECU tune files)

TOTAL: 5 hours (300 minutes)

# CLASSROOM — 2 hours 30 minutes (150 min)

(Demonstrate using any working tune + live Cell Table viewer)

### 1. VE Table Orientation (RPM vs Load) — 50 min

- – VE (volumetric efficiency) table defines how much air the engine is estimated to take in.

- – X-axis is RPM, Y-axis is load (MAP, TPS, or blend).

- – ECU looks up the cell where RPM & load intersect to calculate fuel PW.

- – Show how surrounding cells interpolate to make smooth transitions.

- – Explain steady-state vs transient areas (idle, cruise, accel, decel).

- – Demonstrate how small changes in VE affect AFR immediately.

### 2. Ignition Timing Table Structure — 50 min

- – Timing table is also RPM vs load, but values represent degrees before top dead center (BTDC).

- – Low load = high timing advance; high load = low timing to prevent knock.

- – Idle typically 10–15°, cruise 30–45°, WOT 10–25° depending on engine.

- – Discuss how compression, octane, cam, and boost change timing needs.

- – Explain interpolation and why consistent row/column spacing matters.

### 3. Rich vs Lean vs Knock-Safe — 40 min

- – Lean mixtures improve fuel economy but increase combustion temp.

- – Rich mixtures cool the cylinders and are safer at high load.

- – Stoich (14.7:1) is only for emissions, not for power tuning.

- – Knock occurs when fuel auto-ignites too early → very harmful.

- – Timing and AFR work together: more advance requires safer AFR.

- – Explain “borderline knock timing” vs “safe timing.”

- – Demonstrate expected AFR at idle, cruise, WOT with examples.

### 4. Classroom Q&A — 10 min

- – Final review before hands-on tuning.

Break (30m)

# LAB — 2 hours  (120 min)

### 1. Open a .msq or AlphaECU Tune File (Any file provided by Norman) — 30 min

- – Students load the tune into TunerStudio / AlphaECU software.

- – Identify VE & timing tables in the UI.

- – Validate firmware/tune version compatibility.

- – Check sensor calibration and basic settings.

### 2. Deliberately Change 2 Cells (Fuel + Timing) — 60 min

- – Students choose one VE cell and one ignition cell.

- – Apply a controlled change (example: +5% VE, +3° timing).

- – Save tune as a new file to keep original intact. (Make sure to preserve!)

- – Explain why small changes are safer and easier to evaluate.

- – Discuss how these changes would affect a real engine.

### 3. Compare Logs Before/After + Observe Oscilloscope Behavior — 45 min

- – Load two logs: one with original tune, one with modified cells.

- – Observe AFR reaction from VE change (richer or leaner).

- – Observe timing change effect on ignition dwell and spark event timing.

- – Use oscilloscope to visualize injector PW and ignition timing shift.

- – Students learn how to correlate log data → table changes → waveform changes.

### 4. Lab Review + Cleanup — 15 min

- – Review each student's changes and identify correct/incorrect logic.

- – Restore ECUs to safe base tune.

- – Shut down laptops, power supplies, and store harnesses.