# DAY 6 — STARTUP CAL, AFR TARGETS, BASIC TUNING LOGIC (3 hours total)

### Breakdown

- Classroom: 1 hr 20 min (added cranking section)

- Quiz: 15 min

- Break: 30 min

- Lab: 55 min

# CLASSROOM (1 hr 20 min)

### 1. Cranking Fuel & Cranking Logic (15 min)

- During cranking (typically <300 RPM), normal VE logic does not work well because airflow is extremely low.

- Engines generally start best rich, it’s easier to cobust 10AFR than 17AFR

- Too rich = wet plugs, slower crank speed, “chug but no start.”

- Too lean = most common reason a fresh ECU setup won’t start.

  - Lean mixtures burn slower; at cranking speeds the flame does not propagate fast enough.

  - Lean cranking gives: popping, coughing, no ignition event

- Why injector deadtime matters more at cranking (battery effects have a lot to do with what the injector is doing).

- How CLT affects cranking fuel (cold = even more enrichment required).

- Demonstrate typical cranking pulsewidth values (e.g., 5–20 ms depending on temp, injectors, cam).

### 2. Cold Start Enrichment (10 min)

- Fuel wall-wetting: fuel sticks to intake walls when cold → ECU compensates.

- Cold engines need extra fuel until cylinder walls, injectors, and intake warm up.

- Relationship: colder CLT = higher enrichment curve.

### 3. Warm-Up Enrichment (10 min)

- After the engine starts and idles for a few seconds, cold-start enrichment transitions to warm-up enrichment.

- Warm-up reduces fuel gradually as temperature climbs toward operating temp.

- Usually a small correction until components are hot enough to behave predictably

### 4. AFR Targets Table — Speed Density Logic (15 min)

- VE equation and MAP-based load.

- AFR target defines the goal, VE table defines how much air the engine takes in.

- How AFR table influences injector pulsewidth indirectly.

- Idle, cruise, and WOT AFR strategies.

### 5. Closed-Loop O2 Feedback (15 min)

- Narrowband vs wideband, how trim percentages are applied.

- Why you limit closed-loop authority (e.g., ±10–20%).

- Closed-loop cannot fix:

  - wrong cranking fuel

  - wrong warm-up

  - wrong VE values

  - sensor wiring errors

- STFT/LTFT-like behavior in aftermarket ECUs.

### 6. Startup Cal Sequence Interaction (10 min)

- Cranking fuel → After-start enrichment → Warm-up enrichment

- What after-start enrichment does (extra fuel for 5–20 seconds after engine catches).

- Common tuning mistakes:

  - Lean cranking causing long crank/no start

  - Too much after-start causing rich, lumpy idle

  - Warm-up not tapering correctly → rough running until 120°F+

### 7. Log Analysis for Tuning Decisions (15 min)

- How to identify lean cranking (RPM spikes, no AFR response, no ignition events).

- Spotting sensor issues during startup (CLT wrong → wrong fuel).

- Reading MAP vs RPM to find incorrect VE regions.

# QUIZ (15 min)

Questions about:

- Cranking logic

- AFR target behavior

- Closed-loop limitations

- Startup sequence

- Speed density fundamentals

# BREAK (30 min)

# LAB (55 min)

### 1. Edit AFR Target Table (15 min)

Adjust idle/cruise/WOT targets and explain why richer AFR often stabilizes idle.

### 2. Replay Logs & Diagnose Startup Behavior (20 min)

- Identify lean vs rich cranking in actual logs.

- Look for MAP, RPM, PW, and AFR behavior during start.

### 3. Adjust VE Table Based on Logs (10 min)

Make VE corrections to match targets and fix startup issues.

### 4. Tune Analyzer Demonstration (10 min)

Compare manual vs automatic VE adjustments and discuss when/why to trust each.