# DAY 7 — BASIC TROUBLESHOOTING: “WHY DOESN’T IT START?” (3 hours total)

### Breakdown

- Classroom: 1 hr 15 min

- Quiz: 15 min

- Break: 30 min

- Lab: 1 hr

# CLASSROOM (1 hr 15 min total)

### 1. No-Spark Faults — 20 min

- How to identify spark failure by sound, smell, and lack of combustion.

- Ignition coil quick-test: visible/audible spark check.

- Common causes: bad coil, no 12V, failed driver, missing trigger, blown fuse.

- Why ECU may not command spark (sync loss, RPM = 0).

### 2. No-Fuel Faults — 15 min

- Same no-combustion symptoms as no spark.

- Starting fluid test: If it starts momentarily → spark OK → fuel issue.

- Causes: dead pump, no injector power, clogged injectors, failed driver.

- How to differentiate “fuel but wrong amount” vs “no fuel at all.”

### 3. Trigger Issues — 20 min

- RPM behavior during cranking: jumpy, unstable, or zero RPM.

- Using the oscilloscope to confirm crank/cam signals.

- Hall vs VR typical failures.

- How bad triggers cause both no-spark and no-fuel.

### 4. Ground Reference Faults — 20 min

- How bad grounds show up as sensor failures:

  - TPS stuck or backwards

  - MAP oscillating or incorrect

  - CLT/IAT impossible temps

- Why ground faults can mimic every other problem.

- How to verify with multimeter between ECU GND and chassis GND.

# QUIZ (15 min)

5–10 questions covering:

- Diagnosing spark vs fuel

- Trigger symptoms

- Sensor behavior during ground issues

- Logical troubleshooting order

# BREAK (30 min)

# LAB (1 hr total)

### Introduction to Fault Board — 5 min

Explain the “pre-broken” harness and test setup.

### 1. Bad Ground — 20 min

- Students observe incorrect sensor values.

- Measure ground drop.

- Discover and repair missing ground connection.

### 2. Reversed TPS Pins — 15 min

- Use multimeter to identify 5V, GND, and signal.

- Confirm reversed sweep in TunerStudio.

- Correct wiring.

### 3. One Injector Not Powered — 20 min

- Students check injector 12V feed and coil resistance.

- Confirm dead cylinder via TunerStudio PW and fuel pump output.

- Fix power supply issue.