# DAY 8 — FINAL PRACTICAL / MINIMUM FUNCTIONAL BENCH ECU (3 hours)

### Breakdown

- Classroom:65 min

- Quiz (final theory check): 15 min

- Break: 30 min

- Capstone Lab: 1 hr 30 min

# CLASSROOM (65 min)

### 1. Review of Key Concepts — 40 min

A guided summary of everything needed for a real ECU install:

- Power fundamentals (12V, main relay, fuel pump relay).

- Sensor operation (TPS, MAP, CLT/IAT, crank).

- VE table, AFR targets, warm-up, cranking enrichment basics.

- Trigger logic and no-start diagnostics.

- How to use TunerStudio/AlphaECU reliably.

Goal: make sure students understand why things work, not just how.

### 2. “Tuning vs Wiring” Boundary — 25 min

This is very important for real-world troubleshooting.

- Wiring problem symptoms:

  - Sensors reading impossible values

  - No spark or no fuel

  - ECU doesn’t connect

  - RPM = 0

  - One injector or coil dead

- Tuning problem symptoms:

  - Starts but runs rough

  - Lean/rich areas in log

  - Poor throttle response

  - Bad cold start behavior

  - Knock or weak power in certain cells

Make students memorize the rule:

If the ECU doesn’t see correct sensor values → it’s wiring.
 If sensor values look correct but engine behavior is wrong → it’s tuning.

# QUIZ (15 min)

Short final theory check:

- Identify which issues are wiring vs tuning

- Basic sensor logic

- VE vs AFR target relationship

- Trigger signal expectations

- What a correct bench setup should show in TunerStudio

# BREAK (30 min)

# CAPSTONE LAB (1 hr 30 min total)

Goal: prove the student can build, connect, and operate a minimum viable ECU system on the bench.

### 1. Harness Build — 25 min

Students must independently wire:

- TPS (5V, GND, signal)

- MAP sensor (5V, GND, signal)

- ECU Power (12V, GND, and ignition/enable as required)

Instructor verifies correct pinout and continuity.

### 2. ECU Power-Up & Laptop Connection — 20 min

Students must:

- Power the ECU with their harness

- Connect via USB/serial

- Establish communication in TunerStudio/AlphaECU

- Load the project successfully

### 3. Demonstrate Working Live Values — 20 min

Students must:

- Show TPS % moving

- Show MAP changing (vacuum pump or blowing by mouth)

- Confirm sensors are stable (no noise, no drops, correct range)

This proves wiring, sensor understanding, and ECU communication.

### 4. Tune Modification & Save — 25 min

Students must:

- Open the fuel table

- Make a 2% change in any rpm/load cell

- Burn/save tune to ECU correctly

- Reboot ECU to confirm the change persisted

This proves they understand TunerStudio workflow, ECU memory, and how to make safe tune edits.

# PASS CRITERIA

The student passes if they can:

### ✔ Build a functional TPS + MAP + CKP + power sub-harness

### ✔ Power the ECU on the bench

### ✔ Connect laptop and establish communication

### ✔ Show correct live sensor data

### ✔ Show RPM using a metallic object + crank sensor

### ✔ Modify the tune and save it

Pass = You can actually run an ECU + laptop + hand-built harness on the bench.