## Proposal for Collaboration

### Between AlphaX LLC and Automeca Technical College

### EFI Technician Training and Joint Development Partnership

### 1. Introduction

AlphaX LLC is a Puerto Rico–based company specializing in advanced automotive electronics, including the AlphaECU line of Engine Control Modules and related tuning systems. Our mission is to empower future technicians and engineers through education and innovation in modern automotive technology.

Automeca Technical College has established itself as a leader in technical education and professional development across Puerto Rico, preparing thousands of students for careers in automotive, mechanical, and electrical disciplines.

This proposal seeks to formalize a collaboration between AlphaX and Automeca to deliver a structured EFI (Electronic Fuel Injection) curriculum, provide hands-on learning opportunities, and develop technical projects that advance both education and innovation.

### 2. Purpose of the Collaboration

The purpose of this partnership is to:

- Deliver an industry-relevant EFI curriculum developed by AlphaX.

- Provide hands-on technical training in modern EFI systems and tuning.

- Encourage joint project development in areas such as engine management, fabrication, and motorsport applications.

- Create opportunities for co-sponsorship and brand collaboration that benefit students and enhance both institutions’ visibility.

The program will begin with a 40-hour introductory course, which serves as the foundation of a larger, three-part EFI specialization track totaling 120 hours.

### 3. Program Structure

#### Phase 1 (MVP): EFI Fundamentals — Electronics and Tuning Theory (40 hours)

Goal: Build a solid foundation in how EFI systems work, both electronically and mechanically.
 Core Topics:

- Electrical and electronic basics: voltage, current, resistance, Ohm’s law, sensors, and signal conditioning.

- Automotive electronics: grounding, power distribution, relays, injectors, ignition coils, and key sensors.

- Engine operation fundamentals: air-fuel ratio, ignition timing, volumetric efficiency, and stoichiometry.

- EFI control theory: open vs. closed loop, PID control, fuel and spark maps.

- Introduction to tuning software: TunerStudio or rusEFI examples.

- Lab activities: sensor testing, waveform analysis, and wiring interpretation.

Outcome:
 Students who complete this course will earn the AlphaX Basic EFI Technician Certification, demonstrating the ability to understand, interpret, and apply EFI principles in a professional environment.

### 4. Future Expansion Plan (Full Curriculum Overview)

Once the 40-hour MVP course is successfully launched, AlphaX and Automeca will collaborate to roll out the remaining two courses to complete the 120-hour AlphaX EFI Specialist Program:

| Course | Name | Hours | Focus |
| --- | --- | --- | --- |
| 1 | EFI Fundamentals — Electronics & Tuning Theory | 40 | Theory and diagnostics |
| 2 | EFI Practical — Wiring, Fabrication & ECU Assembly | 40 | Hands-on wiring and hardware |
| 3 | EFI Integration & Advanced Tuning | 40 | Real engine calibration and integration |

### Course 1: EFI Fundamentals — Electronics and Tuning Theory (40 hours)

Goal: Build a solid foundation in how EFI systems work, both electronically and mechanically.

Key Topics:

- Electrical and electronic basics: voltage, current, resistance, Ohm’s law, sensors, signal conditioning.

- Automotive electronics: grounding, power distribution, relays, injectors, ignition coils, O2 sensors, MAP, TPS, etc.

- Engine operation fundamentals: air-fuel ratio, ignition timing, volumetric efficiency, stoichiometry, load calculation.

- EFI control theory: closed-loop vs open-loop, PID control, fuel and spark maps, correction tables.

- Introduction to tuning software: how ECUs interpret data, datalogging, map editing (could use AlphaECU + TunerStudio or rusEFI examples).

- Lab activities: simple sensor testing, voltage and waveform analysis with a scope or simulator.

Outcome: Students understand EFI theory and can interpret sensor data, read wiring diagrams, and understand how maps affect engine behavior.

### Course 2: EFI Practical — Wiring, Fabrication & ECU Assembly (40 hours)

Goal: Develop hands-on competence in building and testing EFI hardware.

Key Topics:

- Harness design and layout: wire gauge, fusing, routing, shielding, grounding strategies.

- Connector and pinout practices: Deutsch, Sumitomo, Molex, OEM connectors.

- Crimping, soldering, and harness sealing techniques.

- Bench testing setups: simulators, power supplies, injectors, ignition drivers.

- Intro to PCB-level understanding: what’s inside an ECU, how signals flow from sensors to drivers.

- Building simple test rigs: for injectors, coils, sensors.

- Safety and reliability testing.

Lab Activities:

- Build a basic EFI harness from schematic.

- Bench-test injector and ignition control circuits.

- Identify and repair wiring faults.

Outcome: Students can independently build and troubleshoot an EFI wiring harness and understand ECU hardware from a practical standpoint.

### Course 3: EFI Integration & Advanced Tuning (40 hours)

Goal: Merge theory and practice into real-world applications.

Key Topics:

- System integration: connecting an ECU to a real engine.

- Startup calibration: setting base maps, crank sync, idle tuning, lambda target tables.

- Dyno tuning or simulated tuning exercises: adjusting fuel, spark, and enrichment tables.

- Diagnostics: reading logs, interpreting error codes, troubleshooting sensors and actuators.

- Flex fuel, boost control, and idle strategies.

- CAN communication and data streaming basics.

- Final project: each student installs and tunes a full EFI system on a bench engine or simulation setup.

Outcome: Students complete a fully functional EFI setup and tune it to stable operation — the bridge from “student” to “practitioner.”

### Overall Program Highlights

- Total: 120 hours (ideal for a full-semester technical certification track)

- Each course could end with a practical evaluation:

  - Course 1 → Theory and sensor identification test.

  - Course 2 → Build a functional harness or bench setup.

  - Course 3 → Tune a running engine or simulator.

- Could lead to an “AlphaX EFI Technician Certification” endorsed by the university and potentially local automotive associations.

## Course 1: EFI Fundamentals — Electronics and Tuning Theory (40 hours)

### Day 1 — Introduction to EFI and Electrical Basics (5 hrs)

Topics:

- Course overview, objectives, and safety.

- Overview of modern EFI systems and how they differ from carbureted systems.

- Basic electrical principles: voltage, current, resistance, Ohm’s law.

- Power, ground, and signal concepts.

- Reading wiring diagrams and schematics.

Lab/Activity:
 Measure voltage, current, and resistance on sample circuits using a multimeter.
 Identify real EFI components (injectors, sensors, coils).

### Day 2 — Automotive Electronics and Sensor Fundamentals (5 hrs)

Topics:

- Overview of automotive sensors: analog vs. digital signals.

- Sensor types and functions: MAP, TPS, IAT, CLT, O2, crank/cam sensors.

- Understanding signal types: 0-5V, frequency, PWM, variable reluctance.

- Sensor grounding and shielding.

Lab/Activity:

- Build and measure a voltage divider circuit.

- Simulate a TPS and observe voltage change on a scope or logger.

### Day 3 — Actuators and ECU Outputs (5 hrs)

Topics:

- Fuel injector operation (saturated vs peak-and-hold).

- Ignition coil drivers and spark generation.

- Idle control, fuel pump relays, and fan relays.

- ECU outputs: low-side vs high-side drivers.

- Basics of PWM control.

Lab/Activity:
 Observe injector and coil driver signals on a scope.
 Test a relay circuit controlled by a simulated ECU output.

### Day 4 — Engine Operation Fundamentals (5 hrs)

Topics:

- The four-stroke engine cycle.

- Combustion fundamentals and air-fuel ratio concepts.

- Volumetric efficiency and load.

- Spark timing and knock.

- Effect of AFR and ignition timing on power and efficiency.

Lab/Activity:
 Use simulation software or TS to visualize AFR, ignition timing, and torque response.

### Day 5 — Fuel Control Theory (5 hrs)

Topics:

- Fuel mass calculation fundamentals.

- Injector flow rate, dead time, and correction tables.

- Speed-density vs. MAF strategies.

- Open-loop vs. closed-loop operation.

- Introduction to O2 feedback control (narrowband vs wideband).

Lab/Activity:
 Calculate required injector size for sample engine builds.
 Simulate AFR response to injector pulse width changes.

### Day 6 — Ignition Control Theory (5 hrs)

Topics:

- Ignition timing concepts: advance, retard, dwell.

- Knock detection and spark correction strategies.

- Crank and cam signal decoding (trigger patterns).

- Ignition maps and load vs RPM axis understanding.

Lab/Activity:
 Plot and interpret a spark map in TS or rusEFI console.
 Experiment with dwell time and observe coil saturation effects (simulation or bench demo).

### Day 7 — ECU Architecture and Software Basics (5 hrs)

Topics:

- ECU architecture overview: inputs, processing, outputs.

- Signal conditioning, ADC, filtering.

- Firmware and calibration data separation.

- Introduction to tuning software interfaces (TS).

- Datalogging and interpreting live data.

Lab/Activity:
 Connect to an ECU or simulator, view live sensor data, and log parameters.
 Identify how each signal correlates with engine behavior.

### Day 8 — Review, Integration, and Assessment (5 hrs)

Topics:

- Review of key EFI concepts: sensors, actuators, maps, control loops.

- Discussion of real-world tuning challenges.

- Preparation for Course 2 (hands-on manufacturing & wiring).

- Final Q&A and evaluation.

Lab/Activity:
 Students diagnose a simulated EFI problem (bad sensor signal, missing ground, incorrect map).
 Short written and practical assessment.

### ✅ Course 1 Outcome

By the end of Day 8, students will:

- Understand how EFI systems work electrically and logically.

- Be comfortable reading and interpreting sensor data.

- Be ready to move into Course 2 (EFI Practical Wiring & Hardware) with confidence.

## 🧰 Course 2: EFI Practical — Wiring, Fabrication & ECU Assembly (40 hours)

Goal: Teach students to design, build, and test EFI wiring systems and components.

### Day 1 — Introduction to EFI Hardware and Wiring Design (5 hrs)

- Overview of what makes a good EFI harness.

- Harness planning: schematics, pinouts, and documentation.

- Understanding ECU connectors and terminals.

- Wire types, gauge, insulation, temperature rating, and automotive standards.

Lab:

- Identify and label wires and connectors from real harnesses.

- Build a simple schematic for a two-injector system.

### Day 2 — Power Distribution and Grounding (5 hrs)

- Power supply design for EFI systems (main relay, fuel pump relay, fusing).

- Proper grounding hierarchy and star grounds.

- Shielding and EMI management.

Lab:

- Assemble a power/ground test board with relays and fuses.

- Test for voltage drops and simulate bad grounds.

### Day 3 — Sensors and Signal Wiring (5 hrs)

- Wiring analog, digital, and frequency-based sensors.

- Signal integrity considerations (twisted pair, shielding).

- Connector pinouts and documentation best practices.

Lab:

- Wire up sensors to a bench ECU or simulator and verify readings.

- Practice terminal crimping and continuity testing.

### Day 4 — Injector and Ignition Circuits (5 hrs)

- High-current circuit design: drivers, load control, and flyback protection.

- Injector types (high impedance vs low impedance).

- Coil driver basics and wiring safety.

Lab:

- Build and test a two-channel injector driver setup.

- Observe injector pulse signals with an oscilloscope.

### Day 5 — Harness Building and Assembly Techniques (5 hrs)

- Harness routing, protection, and strain relief.

- Looming methods (DR-25, PET braid, split tubing).

- Labeling and serviceability.

Lab:

- Begin building a small 4-cylinder EFI harness based on schematic.

### Day 6 — ECU Hardware Overview and Bench Setup (5 hrs)

- What’s inside an ECU: input conditioning, microcontrollers, output drivers.

- CAN bus and serial communications basics.

- Bench testing setups for ECUs.

Lab:

- Power an ECU on the bench, simulate sensor inputs, observe outputs.

- Use CAN analyzer or TunerStudio to confirm communication.

### Day 7 — Troubleshooting and Repair (5 hrs)

- Common wiring issues: open circuits, shorts, and noise.

- Testing continuity and voltage under load.

- How to isolate harness and hardware problems.

Lab:

- Diagnose and repair pre-made harness faults.

- Verify repaired circuits function properly.

### Day 8 — Final Project and Evaluation (5 hrs)

Capstone Lab:

- Each student/team completes and tests a small EFI harness or simulator setup.

- Must power up, communicate with ECU, and show working sensors/injectors.

Wrap-up:

- Discussion on best practices, documentation, and next steps for Course 3.

✅ Outcome: Students can independently design, build, and troubleshoot functional EFI wiring harnesses and understand ECU hardware connections.

## 🧩 Course 3: EFI Integration & Advanced Tuning (40 hours)

Goal: Bring everything together — wiring, theory, and real tuning practice.

### Day 1 — System Integration Overview (5 hrs)

- Review: sensors, actuators, harness, ECU.

- Engine setup workflow (bench → car).

- Safety checks before first power-up.

Lab:

- Connect ECU, harness, and sensors on a test bench.

- Verify communication and signals.

### Day 2 — Base Map Setup and Engine Startup (5 hrs)

- Setting up basic parameters: engine displacement, injector size, ignition mode.

- Cranking fuel, dwell time, trigger setup.

- Idle and warm-up enrichment.

Lab:

- Configure a startup calibration.

- Attempt first startup on a demo engine or simulator.

### Day 3 — Fuel Tuning Fundamentals (5 hrs)

- Understanding VE tables and AFR targets.

- Closed-loop vs open-loop tuning.

- Acceleration enrichment and transient fuel.

Lab:

- Adjust fuel maps based on log data.

- Tune for smooth idle and steady cruise.

### Day 4 — Ignition Tuning Fundamentals (5 hrs)

- Spark advance strategies and knock sensitivity.

- Ignition table structure and load axes.

- Knock control and timing safety margins.

Lab:

- Modify ignition maps and observe effects on torque and smoothness (dyno or simulator).

- Log timing data and interpret knock feedback.

### Day 5 — Advanced Control Strategies (5 hrs)

- Lambda control loops and PID tuning.

- Idle control, boost control, and VVT overview.

- Launch control, rev limits, and cut strategies.

Lab:

- Configure a boost or idle control loop.

- Analyze log data for stability and response.

### Day 6 — Diagnostics and Data Analysis (5 hrs)

- Reading logs to identify mechanical or tuning problems.

- Error codes, limp modes, and safety limits.

- Using data visualization for tuning refinement.

Lab:

- Diagnose issues from pre-recorded data logs.

- Recommend calibration or wiring corrections.

### Day 7 — Flex Fuel & CAN Integration (5 hrs)

- Flex fuel sensors and blending strategies.

- CAN data sharing: gauges, dashes, and secondary controllers.

- Interoperability between systems.

Lab:

- Configure flex fuel compensation on ECU or simulator.

- View CAN data frames and interpret parameters.

### Day 8 — Final Tuning Project & Evaluation (5 hrs)

Capstone Lab:

- Students tune a complete EFI setup (bench or engine dyno) from base map to final calibration.

- Must demonstrate stable idle, load response, and logging.

Wrap-up:

- Review of tuning philosophy and real-world considerations (fuel quality, emissions, reliability).

- Certification assessment.

✅ Outcome: Students can take an engine from “unwired” to “running and tuned,” interpreting data, adjusting maps, and applying best practices safely.